/*

* Revision Control Information

*

* $Source: /users/pchong/CVS/sis/sis/retime/retime.doc,v $

* $Author: pchong $

* $Revision: 1.1.1.1 $

* $Date: 2004/02/07 10:14:49 $

*

*/

int

retime_is_network_retimable(network)

network_t *network;

This routine performs some checks on the network and returns a value

of 1 if the network satisfies the constraints of a retimable circuit

(single-phase, edge-triggered circuit). The exported routines and

data-structured make strict assumptions about the network structure

(single-phase, edge-triggered) and so before calling the other routines

the user should call this routine and check the return value....

re_graph *

retime_network_to_graph(network, size, use_mapped)

network *network;

int size;

int use_mapped;

Build the retiming graph from the network. The nodes of the

retiming graph correspond to the nodes in the network and the

weights on the edges are the number of latches between the nodes.

The parameter "size" is provided for clustreing but this is not

yet used.  If the flag "use_mapped" is used the routine will

generate some additional information to allow it to reconstruct

the mapped network after retiming (see the routine

retime_graph_to_network() below)

int

retime_graph_interface(graph, area_r, delay_r, retime_tol, d_clk, new_c)

re_graph *graph;

double area_r, delay_r;

double retime_tol, d_clk, *new_c;

Routine that encapsulates the retiming procedures for the user

that wants to use the retiming algorithm on a graph. This routine

is able to handle graphs with negative weights and uses the

Lieserson's MILP procedure for retiming. No initial states are

computed. This is due to the fact that negative edge weights are

allowed.

int

retime_graph(network, graph, area_r, delay_r, retime_tol,

d_clk, new_c, lp_flag, min_reg, should_init, can_init)

network_t *network;	/* The network that corrsponds to this graph */

re_graph *graph;	/* graph to be retimed */

double area_r;		/* default area cost for a standard register */

double delay_r;		/* default delay cost for a standard register */

double retime_tol;	/* Tolerance for the binary search */

double d_clk;		/* Desired clock cycle */

double *new_c;		/* The clock cycle after retiming */

int lp_flag;		/* == 1 => Use MILP otherwise Saxe's relaxation */

int min_reg;		/* == 1 => minimize num registers, overrides lp_flag */

int should_init;	/* == 1 => compute_initial_ state

== 2 => compute initial states at all costs */

int *can_int;		/* RETURN: == 1 => could do the initialization */

Retime the sequential network specified by `graph' . Returns a value

1 if latches were moved and 0 if there is no retiming possible.

In the retimed graph, the weight values of the edges w(e) will be

modified to reflect the retimed network.

If the "network" paprameter is not NIL(network_t) and the parameter

"should_init == 1" then the new initial states will be computed for

the retimed graph. If such a computation is feasible then the

flag "can_nit" is set to 1, otherwise it is set to 0. If the flag

"should_init" is set then the user must provide the network, nodes

for which correspond to the retime graph.

The `area_r' parameter specifies the area of each register.

The `delay_r' parameter specifies the delay cost of each register.

Delay cost for a register can be thought of as an estimate for its

setup and hold times.

The "retime_tol" is the tolerance for the binary search and the "d_clk"

specifies the cycle time that the user wants to achieve. The retuned

calue "new_c" refers to the best cycle-time that can be achieved.

This may or may not meet the user desired cycle-time "d_clk".

The user has the option of specifying one of three methods for

retiming. These are governed by the flags lp_flag and min_reg

min_reg	lp_flag		Algorithm used

--------------------------------------

1	anything	MInimize number of registers (uses Simplex)

0	1		UseLieserson's  MILP formulation

0	0		Use Saxe's relaxation algorithm

network_t

retime_graph_to_network(graph, use_mapped)

re_graph *graph;

int use_mapped;

This routine will generate a Boolean network from the specified

graph. The flag "use_mapped" when set to 1 will result in the

routine accessing the nodes in the original network so as to

preserve the mapping of the circuit.

/* Memory management functions */

re_graph *

re_graph_alloc()

Allocate a new graph.  Initialize the array fields to zero

elements.

void

re_graph_free(graph)

re_graph *graph;

Free a graph.

re_graph *

re_graph_dup(graph)

re_graph *graph;

Make a duplicate copy of a graph.

re_edge *

re_create_edge(graph, source, sink, index, weight, breadth)

re_graph *graph;

re_node *source;

re_node *sink;

int index;

int weight;

double breadth;

Allocate a new edge.  The source and sink nodes are assumed

to have been created.  The new edge is created and inserted

into the graph data structure, the list of fanins of the sink

node, and the list of fanouts of the source node. THe connection

is mode to the "index" fanin of if the sink node. The id fields

of the edge structure are properly assigned.  The weight

field is assigned the value `weight'. The breadth of the edge is

the cost of putting a latch on it (may exceed 1 if it represents

a bus).

/* Counting functions */

int

re_num_nodes(graph)

re_graph *graph;

Return the number of nodes in the graph.

int

re_num_edges(graph)

re_graph *graph;

Return the number of edges in the graph.

int

re_num_primary_inputs(graph)

re_graph *graph;

Return the number of primary input nodes in the graph.

int

re_num_primary_outputs(graph)

re_graph *graph;

Return the number of primary output nodes in the graph.

int

re_num_internals(graph)

re_graph *graph;

Return the number of internal nodes in the graph.

Number of internal nodes is equal to the total number

of nodes in the graph minus the primary input nodes

and the primary output nodes.

int

re_num_fanins(node)

re_node *node;

Return the number of fanin edges to node.

int

re_num_fanouts(node)

re_node *node;

Return the number of fanout edges from node.

/* Random access functions */

re_node *

re_get_node(graph, index)

re_graph *graph;

int index;

Return the index'th node of the graph.  The nodes are numbered

starting from 0.  Returns NIL(re_node) if index >= re_num_nodes(graph).

Access should be very quick.

re_edge *

re_get_edge(graph, index)

re_graph *graph;

int index;

Return the index'th edge of the graph.  The edges are numbered

starting from 0.  Returns NIL(re_edge) if index >= re_num_edges(graph).

Access should be very quick.

re_node *

re_get_primary_input(graph, index)

re_graph *graph;

int index;

Return the index'th primary input of the graph.  The primary

inputs are numbered starting from 0.  Returns NIL(re_node)

if index >= re_num_primary_inputs(graph).

Access should be very quick.

re_node *

re_get_primary_output(graph, index)

re_graph *graph;

int index;

Return the index'th primary output of the graph.  The primary

outputs are numbered starting from 0.  Returns NIL(re_node)

if index >= re_num_primary_outputs(graph).

re_edge *

re_get_fanin(node, index)

re_node *node;

int index;

Return the index'th fanin edge of the node.  The fanin

edges are numbered starting from 0.  Returns NIL(re_edge)

if index >= re_num_fanins(node).

re_edge *

re_get_fanout(node, index)

re_node *node;

int index;

Return the index'th fanout edge of the node.  The fanout

edges are numbered starting from 0.  Returns NIL(re_edge)

if index >= re_num_fanouts(node).

/* Generators */

re_foreach_node(graph, index, node)

re_graph *graph;

int index;

re_node *node;

Generate over all nodes in the graph (that is, including

primary inputs and primary outputs).

re_foreach_edge(graph, index, edge)

re_graph *graph;

int index;

re_edge *edge;

Generate over all edges in the graph.

re_foreach_primary_input(graph, index, node)

re_graph *graph;

int index;

re_node *node;

Generate over all primary inputs of a graph.

re_foreach_primary_output(graph, index, node)

re_graph *graph;

int index;

re_node *node;

Generate over all primary outputs of a graph.

re_foreach_fanin(node, index, edge)

re_node *node

int index;

re_edge *edge;

Generate over all the fanin edges to node.

re_foreach_fanout(node, index, edge)

re_node *node

int index;

re_edge *edge;

Generate over all the fanout edges from node.

The generators are macro's.  Typical usage is:

re_foreach_node(graph, index, node) {

/* do something with node */

}

The generator is implemented using the array package.

int

re_ignore_edge(edge)

re_edge *edge;

If the edge is marked as being deleted it is not removed from

the graph but just marked as deleted. This routine can be

used to query for such edges. During traversal these edges

should not be followed.

int

re_ignore_node(node)

re_node *node;

If the node is marked as being deleted (of type RE_IGNORE)

this routine can be used to query for such nodes. During

traversal these nodes should not be visited since some fields

may be out of date.

/* Exportable retiming functions */

int

re_min_fanin_weight(node)

re_node *node;

Return the minimum weight from the fanin edges.

int

re_min_fanout_weight(node)

re_node *node;

Return the minimum weight from the fanout edges.

int

re_max_fanin_weight(node)

re_node *node;

Return the maximum weight from the fanin edges.

int

re_max_fanout_weight(node)

re_node *node;

Return the maximum weight from the fanout edges.

int

re_sum_of_edge_weight(graph)

re_graph *graph;

Return the sum of weights from the edges in the graph.

int

re_effective_sum_edge_weight(graph)

re_graph *graph;

Return the effective sum of weights from the edges in the graph.

Effective meaning that only the `maximum' weight from the

fanout edges of the given node will be counted in the total

weight.  This is useful to model `register sharing'.

double

re_sum_node_area(graph)

re_graph *graph;

Return the sum of area of the nodes.

double

re_total_area(graph, area_r)

re_graph *graph;

double area_r;

Return the sum of area including both area contributed by the

nodes and area contributed by the registers.  This area calculation

uses the register sharing model.  The parameter `area_r' specifies

the area cost of a standard register.

double

re_cycle_delay(graph, delay_r)

re_graph *graph;

double delay_r;

Return the critical delay of the graph.  The parameter `delay_r'

specifies the delay cost of a standard register.  Delay cost is

usually an estimated sum of the setup and hold time.